Titiel: Biogenic Dynamic Material Systems Model – bioDYM – biogenic carbon management

# Introduction

Carbon is a fundamental element for natural and anthropogenic systems. It is unavoidable for nutrition, construction, energy and many other purposes. Biobased, fossil, mineral, gaseous or dissolved carbon forming the global carbon cycle, constantly transitioning between the large carbon stocks: Atmosphere, Lithosphere, hydrosphere and biosphere. The accumulation of carbon in the atmosphere is the major driver for climate change. To decrease the speed of this development, the objective of Carbon neutrality and net Zero until 2050 was formulated by the Paris Agreement in 2015(Source). This objective requires equal anthropogenic carbon output and input. Since the use of carbon remains foreseeable unavoidable, Carbon Dioxide Removal (CDR) approaches like Carbon Capturing and Utilization (CCU) or carbon capturing and storing (CCS) are becoming an important part of climate change mitigation strategies. At the same time photosynthesis is naturally extracting carbon from the atmosphere to form biomass as part of the biosphere. This can be exploited for the targeted sequestration of biogenic carbon by the application of biobased materials as a natural CDR strategy. However, effective and strategic carbon management requires considering the connections to the global carbon cycle as well as advanced tools to analyze large and complex systems in space and time.

Material Flow Analysis (MFA) is an established methodology for the analysis of environmental systems (Brunner & Rechberger, 2004). Various examples for the effective application of MFA for environmental systems are published (Muhayodin et al., 2021) (Fritze & Rotter, 2024). It is based on the law of conservation of mass and evaluates input and outputs of stocks and processes within a system defined in space and time. This makes the MFA a valuable carbon management tool, which can provide system evaluations and valuable insight into the quantity and the efficiency of biogenic carbon sequestration techniques. Carbon management is a long-term task, which shows its effects over long periods of time, and needs to consider dynamic changes in stocks and flows. The dynamic MFA (dMFA) allows modelling of dynamic systems over time and forecasting about effects of strategies and actions in the future. For example (Zhang et al., 2025) uses the dMFA methodology to present the development of bioplastics stocks and researches the amount of biogenic carbon within the build environment in Indonesia (Aryapratama & Pauliuk, 2019).

The high potential of dMFA to research complex environmental systems is hindered by the field of available solutions. Especially specific solutions for modeling biomass and biogenic carbon are lacking, due to specific natural processes with individual mechanics. Existing solutions either do not provide the required functionalities or require a higher level of knowledge e.g. in python. STAN 2.7 is a very established tool. Currently 360 articles cite STAN 2.7 in MFA studies within the context of several environmental studies. Even though the intuitive graphic user interface is helpful, the program lacks the ability to enter large data sets, the implementation of dynamic systems, modelling specific processes like the mineralization of biomass, and in data visualization. Tools, like sankeyMatic (source) and E-Sankey (source) are focused on the visualization of data, without providing the calculation infrastructure. With the publication of the python based framework for open dynamic Systems modelling, the analysis of dynamic systems became more practical (Pauliuk & Heeren, 2020). Various researchers used ODYM for dMFAs in the field of metals (add sources). To make use of the large set of functionalities, ODYM requires basic skills, in python coding, which is not yet standard in environmental sciences. In addition, it lacks adaptations, specific for biomasses.

To address this research gap, the study presents Biogenic Dynamic Material Systems Model – bioDYM, a python-based solution for dMFAs, as dynamic tool for biogenic carbon management. The development has a special focus on usability and accessibility, to support future studies in this field. BioDYM integrates specific processes and mechanisms for biobased materials, which are included in the conceptualization. With solutions for two complex case studies, the study presents the functionalities of bioDYM. Case study answers the question of the impact of the degradation of wheat straw to the carbon stock in the lithosphere. Case study 2 quantifies the contribution of wood as an anthropogenic carbon stock in the build environment.

# Methodology

The presented research is built on several conceptual foundations, commonly used in the field of environmental sciences and industrial ecology. Thus 2.1 contains a summary of these important concepts. The listed concepts are integrated as a specific functionality into bioDYM. 2.2 explains its modular architecture and how the different parts are combined in a workflow. BioDYM is applied in two specific dMFA case studies to solve specific problems in carbon cycle assessment and to present the added possibilities of bioDYM.

## Conceptual Foundations

#### Material Flow Analysis (MFA):

The Material Flow Analysis (MFA) forms a systematic and transparent methodology for investigating the material metabolism of anthropogenic systems. Core builds the mass balance principle, which states (Input -Output = delta Stock) at the element, substance, and material levels. It is designed as a tool to determine the main sources, flows, stocks, and emissions of man-made and natural materials. The methodology and was established by (Baccini & Brunner, 2012; Brunner & Rechberger, 2004) and forms the conceptual framework for dynamic MFAs in general and the foundation for BioDYM.

#### Dynamic Material Flow Analysis (MFA)

The dynamic MFA (dMFA) extends this principles from a static to a dynamic model by the additional dimension of time (Brunner & Rechberger, 2004). The time variable is introduced into the accounting process by the in-use stock and its lifetime distribution (Deng et al., 2023). bioDYM implements these principles and is designed to facilitate the work on complex dMFA, specific for biomass.

Stock Dynamics and Lifetime Modeling:

- The dMFA is the objective of the Open Dynamic Material Systems Model (ODYM) published by (Pauliuk & Heeren, 2020), which is an open source framework for material systems modeling programmed in Python. The description of systems, processes, stocks, flows, and parameters is object-based, which facilitates the development of modular software and testing routines for individual model blocks (Pauliuk & Heeren, 2020).The ODYM Framework forms the code basis for bioDYM and is directly applied in the development.

- A concise, one-sentence definition of the concept. What is it?

- Relevance to BioDYM: A sentence explaining its specific purpose within your tool. Why is it included?

- Key Citation(s): A reference to the foundational literature. Who established this method?

Stock Dynamics and Lifetime Modeling:

- Definition: A concise, one-sentence definition of the concept. What is it?

- Relevance to BioDYM: A sentence explaining its specific purpose within your tool. Why is it included?

- Key Citation(s): A reference to the foundational literature. Who established this method?

#### First-Order Decay (FOMP):

Biomass incorporated in the soil forms a dynamic stock that decomposes over time due to the mineralization, a process caused by microorganisms (Robertson & Paul, 2000). The rate of decomposition is proportional to the amount of biomass and is described with the first order kinetics. To model C mineralization kinetics for complex biomasses, consisting of fast- and slow- decaying fractions, a two pooled approach was developed by (Cayuela et al., 2010). This mineralization principle of biogenic carbon is used by bioDYM to model carbon sequestration capabilities of systems, based on specific biomasses, with the implementation of the specific First Order Mineralization Process (FOMP).

- Definition: A concise, one-sentence definition of the concept. What is it?

- Relevance to BioDYM: A sentence explaining its specific purpose within your tool. Why is it included?

- Key Citation(s): A reference to the foundational literature. Who established this method?(Cayuela et al., 2010)

#### Monte Carlo Simulation:

- Definition: A Monte Carlo Simulation is a commonly used approach A concise, one-sentence definition of the concept. What is it?

- Relevance to BioDYM: A sentence explaining its specific purpose within your tool. Why is it included?

- Key Citation(s): (Papadopoulos & Yeung, 2001)A reference to the foundational literature. Who established this method?

(Pauliuk & Heeren, 2020) indicating the next model developments steps should include routines for Monte-Carlo simulations.

## The BioDYM Tool: Architecture and Workflow

BioDYM is an open-source Python framework built upon ODYM, designed to simplify the dynamic MFA of biogenic systems. The tool follows a structured workflow, from data input to results visualization, as illustrated in Figure X.

### Step 1: System Definition in the BioDYM Data Protocol

Core feature of bioDYM is the specific data infrastructure, delivered within a Excel Data input table called the Biodym Data Protocol (Biodym DP). The table contains the possibility of entering all required system parameters. The bioDYM DP is designed in a long table format, to allow the data input for complex systems. The DP is based on a set of data validation mechanics and a color guiding system, useful for the guidance of users. The bioDYM Dp is split up into several sections each of them containing various pages. Table X shows an overview about the sections of the bioDYM DP.

Table 1: Structure overview of the Biodym DP

| Section | Content | Objective |
| --- | --- | --- |
| 0 | Model Configuration | General configuration of the model. Selectable are the time period of the model, special processes, Uncertainty by Monte Carlo simulation, |
| 1 | Flows | Definition of flows and flow parameters (FM, WC, DM, CC). Definition of Flow Data |
| 2 | Processes | Definition of processes and input/ Output-Flows. Definition of process parameters (TC, Dyn TC, Stocks, Initial Stocks, |
| 3 | Special Processes | Definition of the special parameters DSM and FOMP. |
| 4 | Uncertainty Manager | Definition of uncertainty parameters as input for the Monte Carlo Simulation |
| 5 | Scenario Manager | Definition of parameters to create additional scenarios for result comparison |

#### Model Configuration

The model_configuration allows the set-up of all major settings and system boundaries of bioDYM. The selectable is the timeframe, the selection of special processes and the use and the size of a monte Carlo simulation. A scenario manager allows to set up scenarios for result comparison.

#### Flows

The “1_1_Definition_Flows” introduces all flows of the model and connects them with input and output processes. In addition, the flow properties of water content (WC), Dry mass (DM), and Carbon Content (CC) are defined here. “1_2_Data_Flows” is intended to enter specific flow data for specific flows in defined years. The selection of the year of the flow is key for the dynamic material flow calculation.

#### Processes

“2_1_Definition_Processes” has the objective to define the processes of the model including the process properties. These properties define the process type (Input/ process/ output), the use of Transfer coefficient & Dynamic Transfer coefficient, the presence of an stock or an initial stock and initialize the special processes Dynamic Stock Modelling and FOMP. The individual properties are defined in the pages 2_3_Process_TCs, 2_4_Initial_Stock and 2_5_dynamic_tcs.

#### Special Processes

### Step 2: Model Execution and Calculation Engine

Table X contains an overview about the main workflow of the Biodym execution and the designed calculation engine. The four steps describe the main workflow and the basis for the system architecture. As practical userinterface, a jupyter notebook was selected. Each step contains a separated engine including a testing section, proving the functionality of the part.

| Section | Content | Objective |
| --- | --- | --- |
| 1 | Setup and Data Loading** | Prepare environment and load input data |
| 2 | Calculation & Validation | Execute MFA analysis and verify results |
| 3 | Result visualization | Comprehensive analysis and exploration |
| 4 | Export | Save results and generate documentation |

#### Setup and Data Loading

To ensure the expected high usability of Biodym, the setup and data loading process is limited to the selection of the data upload path of the Biodym-DP in the Biodym-NB. With the initialization of bioDYM, the required packages and functions are loaded and initialized. All study-related parameters like model configuration, flows, processes, are loaded from the excel file, saved as variables and are used for the model calculation in step 2. The settings within the model configuration decide which specific calculations are executed.

#### Calculation & Validation

The calculation engine contains the relevant calculation functions for the standard processes as well as the special processes, including the monte carlo calculation and the scenario calculation. To allow the data validation, the model provides a simple qualitative flow diagram and an dynamic mass balance check, for all elemental layers.

##### Visualization

Biodym contains a large variety of visualization functions for the calculation results. All visualizations are interactive with widgets for the selection of specific processes, dynamic time sliders and export functions. Core is the plot of an dynamic Sankey diagram, which provides an comprehensive system overview. Several visualizations of processes, flows and stocks supporting the interpretation of results. Uncertainties resulting from the monte carlo simulation are displayed, if selected.

#### Export

After the successful execution of the calculation, the results are exported as an excel file and saved within the systems folder.

### Step 3: Results Generation and Output

https://git.nilu.no/impact/plastcycle/plastcycle_mfa

Development

Describe biOdym's architecture, conceptual framework, key functionalities, and implementation details (Python, libraries).

### bioDYM’ s architecture

Section 0: Load Packages

Section 1: Definition of System and relevant aspects

Time, Elements, Test System, Unit

Protocolls -> ERD xls

Process definition, TCs, Flows, Stocks

Upload protocoll

Section 2: Data input into model:

Data Flows

Section 3: MFA Calculation

| ID |  |  |
| --- | --- | --- |
|  | Dynamic Input Modelling | DIM |
|  | Dynamic Stock Modelling | DSM |
|  | First Order mineralization process | FOMP |
|  | Dynamic Transfercoefficients | DynTCs |
|  | Monte Carlo Simulation and Uncertainty | MC |

Mineralization process

Final mass balance check

Final results

Excel report

Section 4: Presentation of results

Sankey diagram

## Case Studies to show application of BioDYM

The following two case studies are designed to demonstrate the application of BioDYM for dynamic carbon cycle assessment in complex biogenic material systems. Basis for the carbon cycle assessment is the assumption that all components of the biogenic substances are received either from the Atmosphere (P0) or from the Environment (P1). The atmosphere is the major source for biogenic carbon, through the transformation of CO2 into biomass by photosynthesis. Every other component of the biomass is aggregated into a single environmental flow, originating in the environment, consisting mainly of water and other nutrients. Water and nutrients are part of the water and nutrient cycle. For the scope of the study, the atmosphere and the environment form stocks, with an unlimited supply of material. This assumption is justified with the focus on carbon flows, with the objective to model the carbon uptake of the system.

Agricultural and woody biomass form relevant flows of biogenic carbon, which are globally present and are increasingly entering the discussion around biogenic carbon management. Publications of [source] show the increasing number of publications in that area. To show the added value and the functions of Biodym, two case studies are added to the study. Case study 1 (CS 1) describes the application of wheat straw. Case study 2 (CS 2) quantifies the carbon sequestration capabilities of woody biomass. Both case studies describe and present in detail the functionalities of bioDYM and the bioDYM data protocol.

### Case Study 1: Carbon cycles within the wheat straw economy

#### Objectives:

Case study 1 (CS1) describes harvest, utilization and EoL treatment of wheat straw resulting from agricultural processes. BioDYM is used to calculate carbon flows and to display dynamic carbon cycles. Specific objectives of CS 1 are:

- To quantify the annual and total carbon uptake from the atmosphere and the development of carbon stocks of the system based on dynamic input data.

- To evaluate the impact of a shift in technology over time with different treatment scenarios.

- To analyze first order mineralization processes and carbon uptake of the soil by composting rye straw.

- To identify most critical parameters influencing the system through a Monte Carlo simulation.

#### Systemboundaries:

The system of CS 1 describes the entire lifecycle of wheat, starting with the raw material extraction, product manufacturing, use-phase and EoL-phase. The system is part of the global carbon cycle and includes the atmosphere and the lithosphere as global carbon stocks. The geographical scope is defined as a representative region in central Europe. The temporal boundary for the analysis covers 25 years, from 2025 to 2050, to incorporate future system dynamics based on historical data.

#### Processes and flows

The model consists of 9 processes building one circular system. Every process is located within the system boundaries of the study and is connected with input and output flows with each other. This structure allows modelling cycles within the global carbon cycle. Table 1 contains an overview of all processes in the system, the process type and a description of the function. In addition, it is indicated, if an uncertainty is used for the MC simulation.

Table 2: Process List case study 1

| ID | Name | Type | Uncertainty | Function |
| --- | --- | --- | --- | --- |
| P0 | Atmosphere | Stock | No | Provides carbon for the cultivation of wheat. |
| P1 | Environment | Stock | No | Provides all other supplements & water for the cultivation of wheat |
| P2 | Cultivation | Process | No | Cultivation of wheat |
| P3 | Harvesting | Process; TC | No | Harvesting of wheat. Split of grains and straw |
| P4 | Grain processing & Consumption | Process; TC | No | Processing wheat grains and consumption of resulting food. |
| P5 | Straw collection/ distribution | Process (DynTC) | Yes (normal distribution) | Collection and distribution of wheat straw into several applications. Dynamic TCs model a shift in technology |
| P6 | Incorporation | Process | No | Direct incorporation of straw residues into agricultural soil. |
| P7 | Animal bedding | Process; TC | No | Use of straw as absorbent bedding material in livestock farming |
| P8 | Lithosphere | Stock; FOMP | Yes (normal distribution) | The soil as large carbon stock, including slow mineralization processes of carbon |

Flows

Case study 1 is based on realistic data combined with assumptions with a highly explorative purpose. All flows are defined as product carbon or environmental flows. Each flow is treated as FM with the dimensions, WC, DM and CC. The detailed data sheets for CS 1 as well as the calculation model are available within the SI.

The quantity of harvested wheat is intended to describe a realistic scenario and the common best practice within the system boundaries. Straw input is material flow data about the annual yield (Yt) of wheat straw from 2025 -2050. Assumed is an agricultural area for wheat cultivation of 1000km² (100.000 ha) in Germany. As a yield for winter wheat in Germany 7.7 Mg/ ha with an annual fluctuation of +- 1.0 mg/ha are assumed (source). In addition, a long-term trend of 0.5% yield increase is included in the assumed flow.

Yt=Ytrend,t+∈t			(1)

Ytrend,t=Ybase*(1+rtrend*t-t0) (2)

∈t~N(0,σY2)

Yt final calculated yield for year t

Ytrend,t is the projected yield for year t based on a linear trend

∈t = is the random fluctuation for year t to simulate natural variability

Central for CS1 is the material composition of wheat straw at harvest status. The used data is based on laboratory experiments in combination with literature data. An overview of the material composition is visible in Table X.

|  |  | Uncertainty parameters |  |  |
| --- | --- | --- | --- | --- |
| Material Fraction | Value | Distribution type |  |  |
| Fresh mass (FM) | 100% | None |  |  |
| Water content (WC) | 14% | Triangular | Min= 10% | Max = 18% |
| Dry mass (DM) | 86% | none |  |  |
| Carbon Content of the DM | 48% | Normal | Mean= 48% | Std. dev. =1% |
| Carbon Content of FM |  | None |  |  |

#### Transfer coefficients

Process transfer coefficients (TCs) determining the output flows of selected processes. The processes are based on the transformation principle of biogenic carbon. Transfer coefficients for the wheat harvest are based on a straw/ grain balance. Straw collection & distribution on DBFZ research data, describing the technical biomass potential for wheat straw in Germany (Source). All other TCs are based on illustrative assumptions detailed in the SI.

#### System diagram

Described information about processes, flows and systemboundaries are visible in Figure 1 System diagram for the wheat straw case study (CS1). Processes are shown as boxes, material flows as arrows, and the system boundary is delineated by the dashed line.

Figure 1: System diagram of CS1. Dynamic Stock Modeling (DSM): irst-Order Mineralization Process (FOMP): You have correctly identified the Lithosphere (P8) Dynamic Transfer Coefficients (DynTC) - Uncertainty and Sensitivity Analysis Setup This section describes the procedure of your Monte Carlo (MC) analysis, not just the uncertain parameters. - Monte Carlo Simulation Setup: You should describe the simulation's execution. This includes: - The number of simulation runs (e.g., "The model was run 1,000 times..."). - The Key Performance Indicators (KPIs) you tracked. What specific outputs did you record from each run to analyze the results? (e.g., "the total carbon stock in the lithosphere in the year 2050"). - Sensitivity Analysis Method: Your objectives state you will "simulate the sensitivity of critical processes." You should specify how you will do this. The most common method, which fits your setup, is to generate scatter plots of the uncertain input parameters against the final KPIs. Parameters that show a strong correlation or pattern are identified as the most sensitive.

### Case Study 2: Carbon sequestration of Woody Biomass

#### Objectives:

Objective of case study 1 is the presentation of the following functions of bioDYM.

- Dynamic Input Modelling (DIM)

- Dynamic Stock Modelling (DSM).

- First Order decay modelling process (FOMP).

- Dynamic Transfercoefficients (DynTC).

- Monte Carlo Simulation (MC).

#### Application:

Case study 1 describes harvest, utilization and EoL treatment of wheat straw resulting from agricultural processes. Within this fictional case study, BiODYM is used to calculate carbon flows and to display carbon cycles. Specific objectives are the quantification of carbon uptake from the atmosphere of the system and the change of the anthropogenic carbon stock. The following functions are used to model complex interactions within the system: 1. DynTCs are used for the simulation of an shift in technologies and an increased efficiency. 2. DSM simulates the in-use stock within the system. 3. FOMP is used to simulate composting of rye straw within the soil. 4. A MC is used to simulate the sensitivity of the system.

is a dynamic MFA of pine Wood for the construction of roofs in residential buildings over a timeframe of 100 years from 1950 - 2050. Researched is the carbon flow originating in a fictional forest with an initial stock. Wood from the forest is processed and used in residential buildings. The DSM model is used to model different life times in the in-use stock of the wood. To simulate a shift in technology, an change in the efficiency of wood processing is assumed, which is displayed by dynamic tcs. After the use phase, the majority of wood enters the EoL displayed by incineration. The

#### Systemboundaries:

Case study 1 is located within the following system boundaries: The whole system is considered to be part of the global carbon cycle and includes the atmosphere and the lithosphere as carbon stocks. The study is located in middle Europe. A timeframe of 100 years, from 1950 until 2050 is used to display the dynamic capabilities of Biodym to combine historic data with forecasts. Within the anthropogenic interaction with the wheat the processes “farming”, “harvest”, “use-phase” and “EoL-phase” are modelled.

Hier definieren Sie präzise, was Teil Ihrer Untersuchung ist und was nicht.

- Prozessebene: Welche Lebenswegphasen und Prozesse sind enthalten? (z.B. "Die Analyse umfasst den Anbau von Roggen, die Ernte, die Trennung von Korn und Stroh, verschiedene Nutzungspfade des Strohs wie Eindeckung oder Kompostierung sowie die End-of-Life-Prozesse. Prozesse wie die Herstellung von Düngemitteln oder Landmaschinen liegen außerhalb der Systemgrenzen (background system).")

- Geografischer Raum: Auf welche Region bezieht sich die Studie? (z.B. "Die Studie modelliert die Flüsse innerhalb Deutschlands.")

- Zeitlicher Rahmen: Welcher Zeitraum wird analysiert? (z.B. "Die dynamische Analyse wird für den Zeitraum von 2025 bis 2050 durchgeführt.")

#### Processes and flows

Dies sind die Bausteine Ihres Modells.

- Prozesse: Listen Sie alle im Modell enthaltenen Prozesse auf (z.B. P0: Atmosphere, P1: Environment, P2: Cultivation, etc.) und beschreiben Sie kurz deren Funktion. Eine Tabelle ist hier oft die übersichtlichste Darstellungsform.

- Flüsse und Stoffe: Definieren Sie, welche Materialien oder Elemente verfolgt werden. (z.B. "Das Modell bilanziert vier Stoffe: die Gesamtmasse des Materials ('material'), Wassergehalt ('WC'), Trockenmasse ('DM') und den Kohlenstoffgehalt der Trockenmasse ('CC').")

| ID | Name | Type | Uncertainty | Function |
| --- | --- | --- | --- | --- |
| P0 | Atmosphere | Stock | No | Provides carbon for the cultivation of biomass. |
| P1 | Environment | Process (TC) | No | Provides all other supplements & water for the cultivation of biomass |
| P2 | Forestry | Process (TC) (Maybe Initial Stock?) Abnahme | No | Cultivation of woody biomass |
| P3 | Processing | Process (TC) | No | Processing of woody biomass |
| P4 | Incineration (BHKW) | Process (TC) | No | Incineration of woody biomass in internal BHKWs |
| P5 | Use-Phase | Process (DynTC) | Yes (normal distribution) | Use phase of wood in |
| P6 | EoL-Sorting | Process (TC) | No | Sorting of EoL Wood. Separation of wood for incineration, pyrolysis and recycling |
| P7 | Pyrolysis | Process (TC) | Yes (normal distribution) | Pyrolysis of EoL Wood and production of biochar |
| P8 | Incineration | Process (TC) | No | Incineration of EoL-Wood in a biomass incineration plant |
| P9 | Lithosphere | Stock; FOMP | Yes (normal distribution) | Incorporation of biochar in the soil and carbon sequestration in the soil |

Table 3: List of flows in Case study 2

| ID | Name | Type | Uncertainty | FM | WC | DM | CC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  | Carbon flow |  |  |  |  |  |
|  |  | Material flow |  |  |  |  |  |

# Results

## Results Case studies

### Results case study 1

### Results case study 2

# Discussion

## Added Value of bioDYM to the ODYM Framework

## Further Development

# Conclusion

- System definition:

  - Multielement MFA analysis

- Calculation

  - Dynamic/ static material input

  - Dynamic/ static Transfer coefficients

  - Initial Stocks

  - Dynamic Stock Modelling

  - First Order decay process modelling

  - Scenario comparison (TC Change)

- Visualization

  - System Visualization

| Dynamic Stock Modelling | DSM |
| --- | --- |
| First Order Process Modelling | FOMP |
| Dynamic TCs | DynTCs |
| Initial Stock | InitS |
| Monte Carlo Simulation | MC |

Objective of the study is the application of the ODYM Framework for the development of the bioDYM framework, that fullfilles the following requirements:

- Specific for the requirements of biomass: Freshmass, WC, Drymass,

- Dynamic stock modelling. Individual life times within the use phase

- First Order Decay of biomaterials

- Dynamic Sankey diagrams, Stock modelling

- Increased Usability. Protocols, Data management: Carbon Cycles

- Monte Carlo Simulation to